## CIE (2012) 10-deg “physiologically-relevant” xyz chromaticity coordinates

### Data key

Columns

- Wavelength (nm)

- x10λ chromaticity coordinate

- y10λ chromaticity coordinate

- z10λ chromaticity coordinate

### Notes

The CIE (2012) 10-deg “physiologically-relevant” x10λ, y10λ, and z10λ  chromaticity coordinates are related to the CIE (2012) 10-deg  x10λ, y10λ and z10(λ) CMFs by:

x10λ=x10(λ)x10λ+y10λ+z10(λ)

y10λ=y10(λ)x10λ+y10λ+z10(λ)

z10λ=z10(λ)x10λ+y10λ+z10(λ)

where x10λ, y10λ, and z10λ are the chromaticity coordinates, and x10λ, y10λ and z10(λ) are the CMFs.

The x10λ, y10λ and z10(λ) CMFs are linear transformations of the cone fundamentals according to:

x10=1.93986443 l10λ- 1.34664359 m10λ+ 0.43044935 s10(λ)

y10λ=0.69283932 l10λ+  0.34967567 m10(λ)

z10λ=2.14687945 s10(λ)

where  l10λ, m10λ and s10(λ) are the Stockman & Sharpe (2000) or CIE (2006) 10-deg cone fundamentals tabulated here.

The proposed CIE standard is also much concerned about defining the precision of the CMFs. Thus, the coefficients of the transformation are defined to 8 decimal places (as in the equations above), the results in linear (energy) units are tabulated to 7 significant figures, and the CMFs must be calculated from the cone fundamentals given in linear (energy) units to 9 sf.  Finally, the chromaticity coordinates are defined to 5 dp.

The functions are provided here at 0.1, 1 and 5 nm steps.

Please note that these are proposals that have yet to be ratified by the full TC 1-36 committee or the CIE.

### References

Stockman, A., & Sharpe, L. T. (2000). Spectral sensitivities of the middle- and long-wavelength sensitive cones derived from measurements in observers of known genotype. Vision Research, 40, 1711-1737.

CIE (2006). Fundamental chromaticity diagram with physiological axes. Parts 1 and 2. Technical Report 170-1. Vienna: Central Bureau of the Commission Internationale de l' Éclairage.